Name: Rameen                                                                                                Roll No: 2023-EE-03

EE-322L Analog and Digital Communication                                        Marks Obtained: ________

Lab Report

Experiment No. 8

FM Demodulation using PLL

- Objective

In this Laboratory exercise you will demodulate a frequency modulated signal using PLL which you have generated in lab 7.

- Technical Background

The technical background for Experiment No. 8, FM Demodulation using PLL, is based on the principles of Frequency Modulation (FM) and the use of a Phase-Locked Loop (PLL) as a frequency-tracking demodulator and frequency-to-voltage converter.

FM Demodulation:

Frequency Demodulation is the process of extracting the original message signal m(t) from a received FM signal.

- In an FM signal, information is encoded in the instantaneous frequency.

- The instantaneous frequency fi(t) varies with the message signal amplitude:

fi(t) = fc + kfm(t)

where fc is the carrier frequency and kf​ is the frequency deviation constant.

- To recover the message, the demodulator performs frequency-to-voltage (F/V) conversion, producing a voltage that is proportional to the instantaneous frequency of the FM signal.

Phase-Locked Loop (PLL) as an FM Demodulator

A PLL is ideal for FM demodulation because it can track the frequency of an incoming signal. The PLL is a feedback control system consisting of three main blocks: Phase Detector (PD), Low-Pass Filter (LPF), and Voltage-Controlled Oscillator (VCO).

### Demodulation Mechanism

The PLL demodulates FM signals using its negative feedback loop:

- Phase Tracking: The PLL is locked to the unmodulated carrier frequency fc

- Frequency Deviation: The input FM signal has an instantaneous frequency that varies according to m(t).

- VCO Correction: The VCO frequency continuously adjusts to maintain phase lock with the input fi(t).

- Signal Recovery: The PD produces an error signal proportional to the frequency difference, and the LPF converts it into a smooth control voltage. This voltage forces the VCO to track the input frequency.

- Output: The voltage across the LPF is proportional to the frequency deviations and represents the recovered message signal m(t), while high-frequency components are suppressed.

### Advantages of PLL Demodulation

- High Linearity: Determined by the VCO's voltage-to-frequency characteristic.

- Noise Immunity: FM signals are less sensitive to amplitude noise, and the PLL tracks frequency, reducing noise effects.

- Low Distortion: Minimal distortion makes PLL demodulators suitable for high-fidelity applications.

- Task-1

  - Description

Perform all steps given in previous lab manual to generate the Frequency modulated signal

  - Circuit, Design and Calculations

The following circuit was implemented:

  - Results and Discussions (with all graphs and snaps)

- Task-2

  - Description

First PLL IC CD4046 is used for FM modulation and second CD4046 is used for demodulation of FM modulated signal. Build the circuit shown in Figure 1 for FM modulation and demodulation. It is not necessary to use the buffer and output low pass filter. Show the spectrum of your FM demodulated signal to the instructor.

  - Circuit, Design and Calculations

  - Results and Discussions (with all graphs, and snaps)

The blue signal is the message signal and the yellow signal is the demodulated signal.

- Conclusion

In this lab exercise, we generated and demodulated a frequency-modulated (FM) signal using two CD4046 Phase-Locked Loop (PLL) ICs—one for modulation and the other for demodulation. By following the steps from the previous lab to create an FM signal and then applying it to the demodulation circuit, we were able to retrieve the original message signal.

The demodulated output showed that the PLL could effectively track the frequency variations in the FM signal and convert them back into amplitude variations, replicating the original input message. Observing the output spectrum confirmed that the PLL successfully locked onto the FM signal and accurately reproduced the modulation signal. This experiment highlights the PLL’s practical utility for FM demodulation and provides insight into the frequency-to-amplitude conversion principle fundamental to FM demodulation applications in communication systems.

Overall, this lab deepened our understanding of PLLs in both modulation and demodulation processes and demonstrated their effectiveness in handling frequency-modulated signals.

Rubrics for Experiment No.7

| Performance | Exceeds expectation (2)/(1) | Meets expectation (1)/(0.5) | Does not meet expectation (0.5)/(0) | Marks |
| --- | --- | --- | --- | --- |
| R1: Realization of Experiment’s Hardware on Breadboard. Marks: 0-1 | The circuit is patched correctly, and safely, with neat connections on the breadboard | The circuit is patched neatly and correctly, but not in a workable form | Incapable to patch the circuit correctly and neatly on breadboard |  |
| R2: Knowledge of theoretical aspects Marks: 0-2 | Has theoretical knowledge required for the experiment | Has partial theoretical knowledge about the experiment | Has no background knowledge about the experiment |  |
| R3: Conducting Hardware Experiment. Marks: 0-1 | All the required tasks are correctly implemented | The required tasks are partially implemented | Unable to implement all the tasks even with guidance |  |
| R4: Demonstrate proper results with justification. Marks: 0-2 | Correct results are provided with required justification | Results are provided with minor errors and/or with little justification | Results are provided with major errors and/or with no justification |  |

Rubrics for Lab Manual No.7

| Performance | Exceeds expectation (0.5)/(0.25) | Meets expectation (-)/(-) | Does not meet expectation (0)/(0) | Marks |
| --- | --- | --- | --- | --- |
| R1: Timely submission Marks: 0- 0.25 | The submission is on time | --- | Late submission |  |
| R2: Report completenes s Marks: 0-0.5 | All relevant calculations, specifications, code, graphs, and results are provided with proper explanation. | All the relevant calculations, specifications, code, graphs and results are provided but with little explanation and justification. | Most of the relevant graphs, results, calculations, specifications, and code are missing, as well as their proper explanation and justification is also missing. |  |
| R3: Error- free writeup Marks: 0- 0.25 | The submitted assignment is without any plagiarism and formatting errors. | Some parts of the submitted assignment contain formatting errors and plagiarized material. | The submitted assignment is mostly plagiarized and contain formatting errors. |  |